SECTION 311300
SELECTIVE TREE REMOVAL AND TRIMMING
PART 1   GENERAL 
1.01
DESCRIPTION

A. Company Qualifications:  The Company performing the Work of this section shall be insured and have a minimum of five (5) years of experience in tree removal and trimming.  The person supervising the Work shall also have a minimum of five (5) years of experience in tree removal and trimming.
B. The work shall consist of the removal and disposal of selected trees including stumps and roots.

1.02
SUBMITTALS

A. Submit detailed experience and qualifications description of tree trimming and removal.  Experience and qualifications package should include a description of the types of equipment and experience that can be provided.
1.03
COORDINATION AND SCHEDULING
A. Coordinate work with the Director’s representative to minimize disruptions and facility operations.  The Director’s Representative shall be notified at least three (3) working days prior to performing the work, and should be provided a schedule for the works progression.

1.04
PROJECT CONDITIONS


A.
Protect existing trees and plants during performance of the Work unless otherwise indicated.  Box trees and plants indicated to remain within the grading limit line with temporary orange construction fencing or solidly constructed wood barricades as required.  Protect root systems from smothering.  Do not store excavated material, or allow vehicular traffic or parking within the canopy drip line.  Restrict foot traffic to prevent excessive compaction of soil over root systems.

B.
Review NYS DEC quarantine order for the districts and the regulations of the movement of materials. http://www.dec.ny.gov/docs/lands_forests_pdf/eab2013quarorder.pdf
PART 2   PRODUCTS (Not Used)
PART 3   EXECUTION
3.01
PREPARATION


A.
Protection:



1.
Prevent damage to buildings, pavement, pipes, conduits, poles and other structures above and below ground that are adjoining or included in the contract area.  Repair damage resulting from the contractor’s negligence.




2.
Protect existing trees and shrubs not to be removed.  Cut back to point of branching all broken branches and skinned areas.  




3.
Store materials and equipment in cleared areas away from tree roots. Prevent employees and equipment from trampling over woodland, existing planting, and established lawns.

3.02
TREES ALONG SECURITY PERIMETER

A.
Cut all trees within the 35 foot wide clear zone indicated.


B.
Top and limb all trees before falling, unless otherwise approved by the Director’s Representative.

C. Fall trees away from security perimeter and cut trunks and branches into 6 ft. sections and lay flat on existing ground.  Do not stack trunks or branches, spread as a single layer to cover slope.

D. Cut all stumps 6 inches above ground.

E. Prune trees outside the 35 ft. clear zone along the security perimeter of all branches that overhang the zone.  All cuts shall be cleanly made with sharp tools, flush with the parent trunk or limb.

3.03
REMOVALS – TREES ENTIRELY

EDIT PARAGRAPHS BELOW TO SUIT PROJECT.

A.
Remove 7 trees where indicated.


B.
Top and limb all trees before falling, unless otherwise approved by the Director’s Representative.

C.
Chip out stumps to a depth of not less than 6 inches below finished grade.  Backfill stump holes with topsoil, and seed.

3.04
PRUNING – INSIDE FACILITY

EDIT PARAGRAPHS BELOW TO SUIT PROJECT.
A.
Prune ten trees inside facility of undesirable wood with the resulting crown shaped to the natural habit of the tree, to a minimum of 20 feet clear from buildings.  Remove all diseased and dead branches, and branches interfering with building exteriors.  All cuts shall be cleanly made with sharp tools, flush with the parent trunk or limb.  

3.05
CLEAN UP – TREES TRIMMED AND REMOVED


A.
Remove and dispose of all logs, tree trimmings, and debris from State property.  Leave Work area in a neat uncluttered condition, where indicated or specified.

3.06
MAINTENANCE AND RESTORATION


A.
Restore grades to indicated levels where settlement or damage due to performance of the Work has occurred.  Correct conditions contributing to settlement or damage.

B.
Restore pavements, walks, curbs, lawns, and other exterior surfaces damaged during performance of the Work to match the appearance and performance of existing corresponding surfaces as closely as practicable.

3.07
WORK AREAS AND PERFORMANCE

A.
The Director’s Representative may limit or restrict work areas and scheduling of the tree trimming and/or removal based upon project progress.

END OF SECTION
Updated 3/18/2014

Edited and/or Printed 4/15/2015
311300 - 3
Project No. xxxxx-C


